Lab Assignment 4

Wireshark and AODV vs DSR

Network Simulation Using NS3

### Introduction

In this lab, you will begin by launching Wireshark to capture and analyze live ARP traffic. Next, you will be introduced to NS-3 (Network Simulator 3), a widely used simulation tool for networking research and education. You will use NS-3 to compare the performance of two routing protocols: Ad hoc On-Demand Distance Vector (AODV) and Dynamic Source Routing (DSR).

## Part 1 – Analyzing ARP using Wireshark

For this part of the lab, use Windows, Linux, or macOS machine (do not use the Raspberry Pi). First, download and install Wireshark from https://www.wireshark.org/download.html. Then follow Activity 1 and Activity 2 in the Wireshark ARP tutorial at https://en.wikiversity.org/wiki/Wireshark/Arp to capture and analyze ARP packets.

Answer the following questions while doing Activity 2:

- Briefly describe the functionality of the ARP protocol. Is ARP always used in modern IP networks? If yes or no, explain why.

- Using your packet capture, fill in the details of the ARP Request and ARP Reply packets in Table 1 below. Also attach screenshots from your capture.

- What is the destination MAC address used when broadcasting an ARP packet?

Table 1 ARP Packet Information

| From |  | To |  |  |
| --- | --- | --- | --- | --- |
| MAC | IP | MAC | IP | ARP Packet Length |

### Part 2 – NS3

In this part, you will use NS-3 to compare the performance of two routing protocols: Ad hoc On-Demand Distance Vector (AODV) and Dynamic Source Routing (DSR).

If you are using the provided virtual machine (VM), NS-3 is already pre-installed. Otherwise, follow the instructions below to install it manually.

### NS-3 Installation Guide

Tested on Ubuntu 22.04

#### 0. Install Dependencies

Before proceeding, install the required packages:

sudo apt-get update

sudo apt-get install g++ python3 wget

#### 1. Download NS-3

wget https://www.nsnam.org/releases/ns-allinone-3.46.1.tar.bz2

#### 2. Extract the Archive

tar xjf ns-allinone-3.46.1.tar.bz2

cd ns-allinone-3.46.1/ns-3.46.1

#### 3. Build NS-3 with Examples and Tests Enabled

Follow the tutorial in the link below to build and test NS-3:
https://www.nsnam.org/docs/release/3.46/tutorial/html/quick-start.html#building-and-testing-ns-3

### 2.1 – AODV vs. DSR

In class, we studied two routing protocols for mobile ad hoc networks: AODV and DSR. These protocols are optimized for different scenarios and network dynamics.

In this lab, you will use NS-3 to empirically compare their performance, specifically throughput under varying network conditions:

- Increasing the mobility speed of nodes

- Changing the number of UDP streams

- Changing the number of nodes in the network

### 2.1.1 Network Simulation Setup

- Number of Nodes: 50

- Mobility Model: Each node performs RandomWalk2dMobility in a rectangular field of size 300 × 1500 meters

- Wireless Communication: All nodes are connected via a wireless channel (e.g., WiFi or 802.11b)

- Traffic Model: By default, the simulation includes 10 UDP streams

#### Simulation Timeline

- Total Duration: 200 seconds

- Warm-up Phase:

  - First 100 seconds: No traffic is transmitted

  - Purpose: Allows nodes to populate routing tables and stabilize topology

- Transmission Phase:

  - At 101 seconds, all UDP clients begin transmission

### 2.1.2 Running the Simulation and Measuring Throughput

You are provided with two NS-3 source files:

- protocol-comparison.cc

- RoutingExperiment.h

These are available on Piazza. Follow the instructions below to run simulations and analyze performance.

#### 🛠️ Setup Instructions

- Place Files in the Correct Directory
Move both files to: ~/NS3/ns-allinone-3.46.1/ns-3.46.1/scratch/

- Run a Simulation

Navigate to the ns-3.46.1 directory and run:

./ns3 configure --disable-examples --disable-tests

./ns3 run "protocol-comparison --protocol=1 --CSVfileName=AODV-pkt.csv --speed=20"

  - --protocol=0 → DSR

  - --protocol=1 → AODV

  - --CSVfileName → Output file

  - --speed → Node movement speed (m/s)

- Modifying UDP Streams and Nodes

  - To change the number of UDP streams, modify the value of nSink in protocol-comparison.cc

  - To change the number of nodes, locate and update the relevant variable(s) in the same file (ECSE 6660 students only)

### 2.1.3 Simulation Tasks

You must compute the average throughput (in bits/sec) between seconds 101 and 199, using the ReceiveRate column from the generated CSV file.

#### Simulations to Run

| Scenario | Variable Changed | Values |
| --- | --- | --- |
| 1 | Node speed | 1, 5, 10, 15, 20 m/s (with 10 UDP streams) |
| 2 | UDP streams | 10, 15, 20 (with 10 m/s speed) |
| 3* | Number of nodes | 20, 50, 100 (with 10 UDP streams and 10 m/s) |

*Case 3 is for ECSE 6660 students only.

### 2.1.4 Output Format

The output CSV file contains:

SimulationSecond, ReceiveRate, PacketsReceived, NumberOfSinks, RoutingProtocol, TransmissionPower, NodeSpeed

You will average ReceiveRate values from SimulationSecond = 101 to 199 to get the throughput for each experiment.

Deliverables

All students are expected to submit a short lab report that includes the following:

For All Students (ECSE 4660 and ECSE 6660 students)

- Answer all the questions in Part1.

- Briefly explain the difference between AODV and DSR.

Summarize their key design differences, including how they handle route discovery and maintenance.

- Tabulate the throughput results based on node speed for the two routing strategies.

Use the following format to report throughput values (in bits per second) for both protocols:

| Node Speed | Throughput of AODV | Throughput of DSR |
| --- | --- | --- |
| 1 |  |  |
| … | … | … |

- Plot throughput vs. node speed

- Plot both AODV and DSR on the same graph.

- Label axes clearly (e.g., X-axis: Node Speed, Y-axis: Throughput).

- Include legend and title.

- Plot throughput vs. number of UDP streams.

Repeat the process from above, this time with varying UDP streams (10, 15, 20) at 10 m/s node speed.

- Discuss:

- Is the throughput a linear function of node speed?

- Provide reasoning based on your observations from the plots and tables.

- Recommendation:

- Based on your analysis, which routing protocol (AODV or DSR) performs better under which conditions?

- Justify your answer with simulation evidence and protocol design considerations.

For ECSE 6660 students only:

- Analyze throughput vs. number of nodes.

With 10 UDP streams and a fixed node speed of 10 m/s, the total number of nodes varies (20, 50, 100).

- Create a table as shown:

| Number of nodes | Throughput of AODV | Throughput of DSR |
| --- | --- | --- |
| 20 |  |  |
| … | … | … |

- Provide a short qualitative explanation for any trends or differences you observe between AODV and DSR as node density increases.

If you have any questions, please turn to, ask on Piazza, or email Lab TA:

·            Chibuikem Ezemaduka (ezemac@rpi.edu)